TYPE INSULETYNG SINGLE PHASE  TRASFORMER FULL POWER

| CODE | IPB00040EE23 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | NORM | EN 61558-1 EN 61558-2-4 |
| CLIENTE |  |  |  |  |  |

TYPE						: associated, not short-circuit resistant

by construction

MOBILITY					: fixed

COOLING					: air/natural

PROTECTION CLASS			: I

INSULATION CLASS			: F

TEMPERATURE CLASS			: F

MAX AMBIENT TEMPERATURE		: +40 °C

PHASES					: 1

INDEX PROTECTION TRANSFORMER	: IP 20

CONNECTION INDEX PROTECTION	: IP 20

TIME OF OPERATION			: continuous

FREQUENCY					: 50/60 Hz

WINDINGS WITH				: Cooper

CORE						: Lamination Steel low losses (1,5W/Kg)

EXECUTION					: vacuum-pressure tropicalized

ELECTROSTATIC SHIELD                            : NO

DIELECTRIC STRENGTH			: 5/2.5 KV

SHORT CIRCUIT INPUT VOLTAGE	: 12.7%

DROP OF VOLTAGE				: 9% (119.9V – 119.9V )

LOSS AT NO LOAD				: 4Watt

LOSS WITH LOAD				: 4Watt

CONTAINMENT BOX			: NO

LAYOUT TERMINAL                                      : 2 SIDE

ELECTRICAL FEATURES

| WINDING | TENSION V | CURRENT | POWER VA |
| --- | --- | --- | --- |
| INPUT | 15/0/-15/230/400 |  | 50 |
| OUTPUT 1 | 110 |  | 25 |
| OUTPUT 2 | 110 |  | 25 |

NOTE: SECONDARY FULL POWER WITH LABEL CONNECTIONS

DIMENSIONAL FEATURES

| WIDTH mm | DEPTH mm | HEIGHT mm | WEIGHT kg |
| --- | --- | --- | --- |
| 90 | 106 | 96 | 1.4 |

PLEASE COMMUNICATE ANY CHANGES, IF REQUIRED, BEFORE ORDER